Prius CAN SID PID supported marked blue

SID 01 response	01 00: 41 00 98 18 80 13	(1001 1000 0001 1000 1000 0000 0001 0011)
01 20: 41 20 80 01 80 01	(1000 0000 0000 0001 1000 0000 0000 0001)
01 40: 41 40 C4 CC 00 00 	(1100 0100 1100 1100 0000 0000 0000 0000)

| SID HEX | PID HEX | n | Description | min | max | unit | calculate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 00 | 4 | PIDs supported |  |  |  | A7...D0 == PID1...PID20 |
| 01 | 01 | 4 | Number of trouble codes and I/M info |  |  |  | Bit encoded. See below. |
| 01 | 02 | 8 | Freeze DTC |  |  |  |  |
| 01 | 03 | 2 | Fuel system status |  |  |  | Bit encoded. See below. |
| 01 | 04 | 1 | Calculated engine load value | 0 | 100 | % | A*100/255 |
| 01 | 05 | 1 | Engine coolant temperature | -40 | 215 | °C | A-40 |
| 01 | 06 | 1 | Short term fuel % trim—Bank 1 | -100 (Rich) | 99.22 (Lean) | % | 0.7812 * (A-128) |
| 01 | 07 | 1 | Long term fuel % trim—Bank 1 | -100 (Rich) | 99.22 (Lean) | % | 0.7812 * (A-128) |
| 01 | 08 | 1 | Short term fuel % trim—Bank 2 | -100 (Rich) | 99.22 (Lean) | % | 0.7812 * (A-128) |
| 01 | 09 | 1 | Long term fuel % trim—Bank 2 | -100 (Rich) | 99.22 (Lean) | % | 0.7812 * (A-128) |
| 01 | 0A | 1 | Fuel pressure | 0 | 765 | kPa (gauge) | A*3 |
| 01 | 0B | 1 | Intake manifold pressure | 0 | 255 | kPa (absolute) | A |
| 01 | 0C | 2 | Engine RPM | 0 | 16,383.75 | rpm | ((A*256)+B)/4 |
| 01 | 0D | 1 | Vehicle speed | 0 | 255 | km/h | A |
| 01 | 0E | 1 | Timing advance | -64 | 63.5 | ° relative to #1 cylinder | A/2 - 64 |
| 01 | 0F | 1 | Intake air temperature | -40 | 215 | °C | A-40 |
| 01 | 10 | 2 | MAF air flow rate | 0 | 655.35 | g/s | ((256*A)+B) / 100 |
| 01 | 11 | 1 | Throttle position | 0 | 100 | % | A*100/255 |
| 01 | 12 | 1 | Sec.(?) air status |  |  |  | Bit encoded. See below. |
| 01 | 13 | 1 | Oxygen sensors present |  |  |  | [A0..A3] == Bank 1, Sensors 1-4. [A4..A7] == Bank 2... |
| 01 | 14 | 2 | Bank 1, Sensor 1: Oxygen sensor voltage, Short term fuel trim | 0 0 | 1.275 99.2 | Volts % | A * 0.005 (B-128) * 0.7812 (if B==0xFF, sensor is not used in trim calc) |
| 01 | 15 | 2 | Bank 1, Sensor 2: Oxygen sensor voltage, Short term fuel trim | 0 0 | 1.275 99.2 | Volts % | A * 0.005 (B-128) * 0.7812 (if B==0xFF, sensor is not used in trim calc) |
| 01 | 16 | 2 | Bank 1, Sensor 3: Oxygen sensor voltage, Short term fuel trim | 0 0 | 1.275 99.2 | Volts % | A * 0.005 (B-128) * 0.7812 (if B==0xFF, sensor is not used in trim calc) |
| 01 | 17 | 2 | Bank 1, Sensor 4: Oxygen sensor voltage, Short term fuel trim | 0 0 | 1.275 99.2 | Volts % | A * 0.005 (B-128) * 0.7812 (if B==0xFF, sensor is not used in trim calc) |
| 01 | 18 | 2 | Bank 2, Sensor 1: Oxygen sensor voltage, Short term fuel trim | 0 0 | 1.275 99.2 | Volts % | A * 0.005 (B-128) * 0.7812 (if B==0xFF, sensor is not used in trim calc) |
| 01 | 19 | 2 | Bank 2, Sensor 2: Oxygen sensor voltage, Short term fuel trim | 0 0 | 1.275 99.2 | Volts % | A * 0.005 (B-128) * 0.7812 (if B==0xFF, sensor is not used in trim calc) |
| 01 | 1A | 2 | Bank 2, Sensor 3: Oxygen sensor voltage, Short term fuel trim | 0 0 | 1.275 99.2 | Volts % | A * 0.005 (B-128) * 0.7812 (if B==0xFF, sensor is not used in trim calc) |
| 01 | 1B | 2 | Bank 2, Sensor 4: Oxygen sensor voltage, Short term fuel trim | 0 0 | 1.275 99.2 | Volts % | A * 0.005 (B-128) * 0.7812 (if B==0xFF, sensor is not used in trim calc) |
| 01 | 1C | 1 | OBD standards this vehicle conforms to |  |  |  | Bit encoded. See below. |
| 01 | 1D | 1 | Oxygen sensors present |  |  |  | Similar to PID 13, but [A0..A7] == [B1S1, B1S2, B2S1, B2S2, B3S1, B3S2, B4S1, B4S2] |
| 01 | 1E | 1 | Auxiliary input status |  |  |  | A0 == Power Take Off (PTO) status (1 == active) [A1..A7] not used |
| 01 | 1F | 2 | Run time since engine start | 0 | 65,535 | seconds | (A*256)+B |
| 01 | 20 | 4 | PIDs supported 21-40 |  |  |  | Bit encoded [A7..D0] == [PID 0x21..PID 0x40] |
| 01 | 21 | 2 | Distance traveled with malfunction indicator lamp (MIL) on | 0 | 65,535 | km | (A*256)+B |
| 01 | 22 | 2 | Fuel Rail Pressure (relative to manifold vacuum) | 0 | 5177.265 | kPa | ((A*256)+B) * 0.079 |
| 01 | 23 | 2 | Fuel Rail Pressure (diesel) | 0 | 655350 | kPa (gauge) | ((A*256)+B) * 10 |
| 01 | 24 | 4 | O2S1_WR_lambda(1): Equivalence Ratio Voltage | 0 0 | 2 8 | N/A V | ((A*256)+B)*0.0000305 ((C*256)+D)*0.000122 |
| 01 | 25 | 4 | O2S2_WR_lambda(1): Equivalence Ratio Voltage | 0 0 | 2 8 | N/A V | ((A*256)+B)*0.0000305 ((C*256)+D)*0.000122 |
| 01 | 26 | 4 | O2S3_WR_lambda(1): Equivalence Ratio Voltage | 0 0 | 2 8 | N/A V | ((A*256)+B)*0.0000305 ((C*256)+D)*0.000122 |
| 01 | 27 | 4 | O2S4_WR_lambda(1): Equivalence Ratio Voltage | 0 0 | 2 8 | N/A V | ((A*256)+B)*0.0000305 ((C*256)+D)*0.000122 |
| 01 | 28 | 4 | O2S5_WR_lambda(1): Equivalence Ratio Voltage | 0 0 | 2 8 | N/A V | ((A*256)+B)*0.0000305 ((C*256)+D)*0.000122 |
| 01 | 29 | 4 | O2S6_WR_lambda(1): Equivalence Ratio Voltage | 0 0 | 2 8 | N/A V | ((A*256)+B)*0.0000305 ((C*256)+D)*0.000122 |
| 01 | 2A | 4 | O2S7_WR_lambda(1): Equivalence Ratio Voltage | 0 0 | 2 8 | N/A V | ((A*256)+B)*0.0000305 ((C*256)+D)*0.000122 |
| 01 | 2B | 4 | O2S8_WR_lambda(1): Equivalence Ratio Voltage | 0 0 | 2 8 | N/A V | ((A*256)+B)*0.0000305 ((C*256)+D)*0.000122 |
| 01 | 2C | 1 | Commanded EGR | 0 | 100 | % | 100*A/255 |
| 01 | 2D | 1 | EGR Error | -100 | 99.22 | % | A*0.78125 - 100 |
| 01 | 2E | 1 | Commanded evaporative purge | 0 | 100 | % | 100*A/255 |
| 01 | 2F | 1 | Fuel Level Input | 0 | 100 | % | 100*A/255 |
| 01 | 30 | 1 | # of warm-ups since codes cleared | 0 | 255 | N/A | A |
| 01 | 31 | 2 | Distance traveled since codes cleared | 0 | 65,535 | km | (A*256)+B |
| 01 | 32 | 2 | Evap. System Vapor Pressure | -8,192 | 8,192 | Pa | ((A*256)+B)/4 - 8,192 |
| 01 | 33 | 1 | Barometric pressure | 0 | 255 | kPa (Absolute) | A |
| 01 | 34 | 4 | O2S1_WR_lambda(1): Equivalence Ratio Current | 0 -128 | 2 128 | N/A mA | ((A*256)+B)*0.0000305 ((C*256)+D)*0.00391 - 128 |
| 01 | 35 | 4 | O2S2_WR_lambda(1): Equivalence Ratio Current | 0 -128 | 2 128 | N/A mA | ((A*256)+B)*0.0000305 ((C*256)+D)*0.00391 - 128 |
| 01 | 36 | 4 | O2S3_WR_lambda(1): Equivalence Ratio Current | 0 -128 | 2 128 | N/A mA | ((A*256)+B)*0.0000305 ((C*256)+D)*0.00391 - 128 |
| 01 | 37 | 4 | O2S4_WR_lambda(1): Equivalence Ratio Current | 0 -128 | 2 128 | N/A mA | ((A*256)+B)*0.0000305 ((C*256)+D)*0.00391 - 128 |
| 01 | 38 | 4 | O2S5_WR_lambda(1): Equivalence Ratio Current | 0 -128 | 2 128 | N/A mA | ((A*256)+B)*0.0000305 ((C*256)+D)*0.00391 - 128 |
| 01 | 39 | 4 | O2S6_WR_lambda(1): Equivalence Ratio Current | 0 -128 | 2 128 | N/A mA | ((A*256)+B)*0.0000305 ((C*256)+D)*0.00391 - 128 |
| 01 | 3A | 4 | O2S7_WR_lambda(1): Equivalence Ratio Current | 0 -128 | 2 128 | N/A mA | ((A*256)+B)*0.0000305 ((C*256)+D)*0.00391 - 128 |
| 01 | 3B | 4 | O2S8_WR_lambda(1): Equivalence Ratio Current | 0 -128 | 2 128 | N/A mA | ((A*256)+B)*0.0000305 ((C*256)+D)*0.00391 - 128 |
| 01 | 3C | 2 | Catalyst Temperature Bank 1, Sensor 1 | -40 | 6,513.5 | °C | ((A*256)+B)/10 -40 |
| 01 | 3D | 2 | Catalyst Temperature Bank 2, Sensor 1 | -40 | 6,513.5 | °C | ((A*256)+B)/10 -40 |
| 01 | 3E | 2 | Catalyst Temperature Bank 1, Sensor 2 | -40 | 6,513.5 | °C | ((A*256)+B)/10 -40 |
| 01 | 3F | 2 | Catalyst Temperature Bank 2, Sensor 2 | -40 | 6,513.5 | °C | ((A*256)+B)/10 -40 |
| 01 | 40 | 4 | PIDs supported 41-60 (?) |  |  |  | Bit encoded [A7..D0] == [PID 0x41..PID 0x60] (?) |
| 01 | 41 | ? | Monitor status this drive cycle | ? | ? | ? | ? |
| 01 | 42 | 2 | Control module voltage | 0 | 65.535 | V | ((A*256)+B)/1000 |
| 01 | 43 | 2 | Absolute load value | 0 | 25696 | % | ((A*256)+B)*100/255 |
| 01 | 44 | 2 | Command equivalence ratio | 0 | 2 | N/A | ((A*256)+B)*0.0000305 |
| 01 | 45 | 1 | Relative throttle position | 0 | 100 | % | A*100/255 |
| 01 | 46 | 1 | Ambient air temperature | -40 | 215 | °C | A-40 |
| 01 | 47 | 1 | Absolute throttle position B | 0 | 100 | % | A*100/255 |
| 01 | 48 | 1 | Absolute throttle position C | 0 | 100 | % | A*100/255 |
| 01 | 49 | 1 | Accelerator pedal position D | 0 | 100 | % | A*100/255 |
| 01 | 4A | 1 | Accelerator pedal position E | 0 | 100 | % | A*100/255 |
| 01 | 4B | 1 | Accelerator pedal position F | 0 | 100 | % | A*100/255 |
| 01 | 4C | 1 | Commanded throttle actuator | 0 | 100 | % | A*100/255 |
| 01 | 4D | 2 | Time run with MIL on | 0 | 65,535 | minutes | (A*256)+B |
| 01 | 4E | 2 | Time since trouble codes cleared | 0 | 65,535 | minutes | (A*256)+B |
| 01 | C3 | ? | ? | ? | ? | ? | Returns numerous data, including Drive Condition ID and Engine Speed* |
| 01 | C4 | ? | ? | ? | ? | ? | B5 is Engine Idle Request B6 is Engine Stop Request* |
| 02 | 02 | 2 | Freeze frame trouble code |  |  |  | BCD encoded, see below. |
| 03 | N/A | n*6 | Request trouble codes permanent errors |  |  |  | 3 codes per message frame, BCD encoded. See below. |
| 04 | N/A | 0 | Clear trouble codes / Malfunction indicator lamp (MIL) / Check engine light |  |  |  | Clears all stored trouble codes and turns the MIL off. |
| 05 |  |  | Request oxygen sensor monitoring test results |  |  |  |  |
| 06 |  |  | Request on-board monitoring test results for non-continously monitored systems |  |  |  |  |
| 07 | N/A | n*6 | Request trouble codes intermediate errors |  |  |  | 3 codes per message frame, BCD encoded. See below. |
| 08 |  |  | Request control of on-board system, test or component |  |  |  |  |
| 09 | 01 |  | Anzahl an Antworten |  |  |  |  |
| 09 | 02 | 5x5 | Vehicle identification number (VIN) |  |  |  | Returns 5 lines, A is line ordering flag, B-E ASCII coded VIN digits. |
| 09 | 03 |  | Anzahl an Antworten |  |  |  |  |
| 09 | 04 | 8 | Vehicle identification number (VIN) |  |  |  | Returns 5 lines, A is line ordering flag, B-E ASCII coded VIN digits. |
| 09 | 05 |  | Anzahl an Antworten |  |  |  |  |

SID 21 response: 	21 00: 61 00 BE 1F A8 13 	(1011 1110 0001 1111 1010 1000 0001 0011)
                     98 1A 80 13  	(1001 1000 0001 1010 1000 0000 0001 0011)
21 20: 61 20 80 01 80 01	(1000 0000 0000 0001 1000 0000 0000 0001)
21 40: 61 40 C4 CC 00 01	(1100 0100 1100 1100 0000 0000 0000 0001)
21 60: 61 60 00 00 00 01	(0000 0000 0000 0000 0000 0000 0000 0001)
21 80: 61 80 00 00 00 01	(0000 0000 0000 0000 0000 0000 0000 0001)
21 A0: 61 A0 00 00 07 C1	(0000 0000 0000 0000 0000 0111 1100 0001)
                       00 00 67 C1	(0000 0000 0000 0000 0110 0111 1100 0001)
21 C0: 61 C0 F7 F0 20 01	(1111 0111 1111 0000 0010 0000 0000 0001)
                       C0 08 06 01	(1100 0000 0000 1000 0000 0110 0000 0001)
21 E0: 61 E0 E6 25 BC C8	(1110 0110 0010 0101 1011 1100 1100 1000)
                       E0 00 00 04	(1110 0000 0000 0000 0000 0000 0000 0100)
                       C0 00 00 04	(1100 0000 0000 0000 0000 0000 0000 0100)

| SID HEX | PID HEX | n | Description | min | max | unit | calculate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | 00 | 4 | PIDs supported |  |  |  | A7...D0 == PID1...PID20 |
|  | 01 |  |  |  |  |  |  |
|  | 03 |  | depend on condition |  |  |  |  |
|  | 04 |  |  |  |  |  |  |
|  | 05 |  |  |  |  |  |  |
|  | 06 |  | depend on condition |  |  |  |  |
|  | 07 |  | depend on condition |  |  |  |  |
|  | 0C |  |  |  |  |  |  |
|  | 0D |  |  |  |  |  |  |
|  | 0E |  | depend on condition |  |  |  |  |
|  | 0F |  |  |  |  |  |  |
|  | 10 |  | depend on condition |  |  |  |  |
|  | 11 |  |  |  |  |  |  |
|  | 13 |  | depend on condition |  |  |  |  |
|  | 15 |  | depend on condition |  |  |  |  |
|  | 1C |  |  |  |  |  |  |
|  | 1F |  |  |  |  |  |  |
|  | 20 | 4 | PIDs supported |  |  |  | A7...D0 == PID21...PID40 |
|  | 21 |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |
|  | 31 |  |  |  |  |  |  |
|  | 40 | 4 | PIDs supported |  |  |  | A7...D0 == PID41...PID60 |
|  | 41 |  |  |  |  |  |  |
|  | 42 |  |  |  |  |  |  |
|  | 46 |  |  |  |  |  |  |
|  | 49 |  |  |  |  |  |  |
|  | 4A |  |  |  |  |  |  |
|  | 4D |  |  |  |  |  |  |
|  | 4E |  |  |  |  |  |  |
|  | 60 | 4 | PIDs supported |  |  |  | A7...D0 == PID61...PID80 |
|  | 80 | 4 | PIDs supported |  |  |  | A7...D0 == PID81...PIDA0 |
|  | A0 | 4 | PIDs supported |  |  |  | A7...D0 == PIDA1...PIDC0 |
|  | B2 |  | depend on condition |  |  |  |  |
|  | B3 |  | depend on condition |  |  |  |  |
|  | B6 |  |  |  |  |  |  |
|  | B7 |  |  |  |  |  |  |
|  | B8 |  |  |  |  |  |  |
|  | B9 |  |  |  |  |  |  |
|  | BA |  |  |  |  |  |  |
|  | C0 | 4 | PIDs supported |  |  |  | A7...D0 == PIDC1...PIDE0 |
|  | C1 |  |  |  |  |  |  |
|  | C2 |  |  |  |  |  |  |
|  | C3 |  | depend on condition |  |  |  |  |
|  | C4 |  | depend on condition |  |  |  |  |
|  | C6 |  | depend on condition |  |  |  |  |
|  | C7 |  | depend on condition |  |  |  |  |
|  | C8 |  | depend on condition |  |  |  |  |
|  | C9 |  | depend on condition |  |  |  |  |
|  | CA |  | depend on condition |  |  |  |  |
|  | CB |  | depend on condition |  |  |  |  |
|  | CC |  | depend on condition |  |  |  |  |
|  | CD |  | depend on condition |  |  |  |  |
|  | D3 |  | depend on condition |  |  |  |  |
|  | D6 |  | depend on condition |  |  |  |  |
|  | D7 |  | depend on condition |  |  |  |  |
|  | E0 | 4 | PIDs supported |  |  |  | A7...D1 == PIDE1...PIDFF |
|  | E1 |  |  |  |  |  |  |
|  | E2 |  |  |  |  |  |  |
|  | E3 |  | depend on condition |  |  |  |  |
|  | E6 |  | depend on condition |  |  |  |  |
|  | E7 |  | depend on condition |  |  |  |  |
|  | EB |  | depend on condition |  |  |  |  |
|  | EE |  | depend on condition |  |  |  |  |
|  | F0 |  | depend on condition |  |  |  |  |
|  | F1 |  | depend on condition |  |  |  |  |
|  | F3 |  | depend on condition |  |  |  |  |
|  | F4 |  | depend on condition |  |  |  |  |
|  | F5 |  | depend on condition |  |  |  |  |
|  | F6 |  | depend on condition |  |  |  |  |
|  | F9 |  | depend on condition |  |  |  |  |
|  | FA |  | depend on condition |  |  |  |  |
|  | FD |  | depend on condition |  |  |  |  |
|  | FE |  | depend on condition |  |  |  |  |